- Load pointcloud, filter frame

- Make spacing exactly 0.25s

- Extract profile

  - Make 5s windows (per profile, lidar angle)

  - Iteratively remove closest point if std>1cm

  - R=Rmean if >50% remaining, else nan

- Remove invalid profiles

  - x(0) < x_min
x_min=792 for S1|S3|storm1, else 796

  - gap along arc length > 0.5 m

- Toe estimation

  - Resample pointclouds to 30s interval. 
Use nearest valid measurement in time

  - Remove (fill) overhang

  - Resample each profile in space: 5 cm spacing along arc length

  - Spatial smoothing: gaussian with sigma=20 cm along arc length

  - Find toe, crest: slope>slope_limit?

    - toe: first point with A) slope>slope_limit and B) avg_slope > slope_limit over the next m

    - crest: last point with A) slope>slope_limit and B) avg_slope > slope_limit over the last m

    - slope limit: S3 15° (no cliff), S1/S2/S4: 20°

  - Temporal smoothing of toe and crest location: Gaussian with sigma=5min

- Find slumping events: decrease in dune volume above z=z_min

  - If (z_crest – z_toe)<1m, z_min = z_toe + 0.3m.

  - Else z_min = z_toe + 0.5 m

  - Slumping if ΔV above z_min > 0.04 m3/m

- Remove foam: temporal filtering

  - Make timeseries of z and intensity (per profile, lidar angle)

  - Make windows of x minutes, centered on t.

  - Shift window edges when it contains slumping events

  - For timesteps where no slumping occurred, filter outliers:

    - Remove worst outliers: z-z_mean > 1 cm & z-z_mean > 2 std & I-I_mean > 1 std

    - Second pass: remove outliers: z-z_mean > 1 cm & z-z_mean > 1 std & I-I_mean > 1 std

- Fills missing values: use previous valid value, unless slumping event in between. Then use next valid value.

- Re-calculate toe and crest based on filtered profiles

  - No spatial smoothing, gaussian smoothing in time with sigma=1min.